# Examples

### LGC FILE

*TITR

Any number of lines for a title

*OLOC # options should be as before

*PUNC

*POIN # points as before

P2	0.0	0.0	0.0

P3	1	1	1

P4	2	2	2

*CALA

P1	0	0	0

*INSTR 	% list of instruments

% instrID  defaultTargetID   instrHt    sigmaInstrHt  sigmaInstrCentering  constAngl

*POLAR   Theo1  T1   0.19345    0.05  0.05  0.0

% targetID    sigmaAngl   sigmaZenD   sigmaDSpt   ppmDSpt   distCorrectionKnown    %distCorrectionValue    sigmaDCorr    sigmaTargetCentering   targetHt    sigmaTargetHt

T1   5.0  5.0  0.5  0.5  true  0.0  0.2  0.05  0.0  0.05

T2   10.0  10.0  1.0  1.0  false  0.0  0.2  0.05  0.0  0.05

*POLAR   LTD1  ccr1.5   0.0    0.00  0.00  0.0

ccr1.5   10.0  10.0  0.03  0.03  true  0.0  0.0  0.00  0.0  0.01

*LEVEL   Niv1  Staff1   	  	% instrID  defaultscaleID

% targetID    sigmaDSpt   ppmDSpt   distCorrectionValue   sigmaDCorr    sigmaTargetCentering targetHt    %sigmaTargetHt

Staff1   0.01  0.01  0.07   0.0   0.05   0.00  0.05

% targetID   sigmaDSpt   ppmDSpt   distCorrectionValue   sigmaDCorr    sigmaInstrCentering

*SCALE   Regle15   1.0  0.0  0.0   0.0  0.5

*SCALE   Ecarto1   0.05  0.01  0.01   0.001  0.5

*TSTN	P1   Theo1        		% instrument height unknown, default target, and instrument parameters used

*V0                         	% whitespace is insignificant

*ANGL	                        		%s.e. and other parameters from instr/target pair

P2	100	OBSE  15	 % specific obs s.e. set

P3	200

P2	101			% this will no longer give an error, repeat obs allowed

*ECTH	200   Regle15      		% s.e. and other parameters from instr or scale

P2	0.05

*V0                            	% these have a different V0 to those above

*ANGL	20

P2	99

P3	201

P2	100          	 % this will no longer give an error

*TSTN	P2    Theo1       IHFIX		% instrument height known, default value used

*V0        ACST     10.0       	% hz. Angle const set to 10 cc

*ZEND

P1	100

P3	200

*PLR3D

P1   100  100  10       TH 0.07      	% HA ZD DI, target height different for this observation

* TSTN	P4	LTD1    ROT3D     	% 3D orientation to calculate, IH fixed to 0.0

*V0

*PLR3D

P1   100   100   10

P2   101   101   11

P3   202   102   12

* TSTN	P1	Theo1           		% 3D orientation to calculate, IH fixed to 0.0

*V0

*ZEND

P2	100      OBSE  10   	% specific obs s.e. set

*DIST

P2	10				% uses default target

P3	20	TRGT T2		% new target and default parameters for the target

P4	30				%  uses the same target as the one above

*PLR3D					% uses default target again

P2   100   100   10   TRGT   T2   ZSE  8    	% new target and default parameters for the target

P3   200   200   20			%  uses the same target as the one above

* TSTN	P2	Theo1      IHFIX     IH  0.19    		% instrument height fixed and value set

*V0

*DIST

P1	10	OBSE 1.0   PPM  1.0   	% specific error data set

P3	20

P4	20

* TSTN	P3	Theo 1    IHFIX    TRGT  T2       	% target set to be T2

*V0

*DIST

P1	10

*ECHO 	  Ecarto1				% new stretched wire with each keyword

P3	0.25

*END

### LGC FILE

*TITR

Any number of lines for a title

*OLOC 		% options should be as before

*PUNC

*POIN 		% points as before

P2	0.0	0.0	0.0

P3	1	1	1

P4	2	2	2

*CALA

P1	0	0	0

*INSTR 	% list of instruments

*POLAR     Theo1     T1      0.19345       0.05     0.05     0.0

T1               5.0     5.0     0.5     0.5     true     0.0     0.2     0.05     0.0     0.05

T2             10.0   10.0     1.0     1.0    false     0.0     0.2     0.05     0.0     0.05

*POLAR     LTD1     ccr1.5          0.0       0.00     0.00     0.0

ccr1.5       10.0   10.0   0.03    0.03     true     0.0    0.0     0.00      0.0     0.01

*LEVEL   Niv1  Staff1

Staff1      0.01   0.01  0.07      0.0     0.05   0.00  0.05

*SCALE      Regle15    1.0      0.0       0.0     0.0    0.5

*SCALE       Ecarto1   0.05   0.01     0.01   0.001  0.5

*TSTN	P1   Theo1        		% instrument height unknown, default target, and instrument parameters used

*V0                         	% whitespace is insignificant

*ANGL	                        		%s.e. and other parameters from instr/target pair

P2	100	OBSE  15	 % specific obs s.e. set

P3	200

P2	101			% this will no longer give an error, repeat obs allowed

*ECTH	200   Regle15      		% s.e. and other parameters from instr or scale

P2	0.05

*V0                            			% these have a different V0 to those above

*ANGL	20

P2	99

P3	201

P2	100          		 % this will no longer give an error

*TSTN	P2    Theo1       IHFIX			% instrument height known, default value used

*V0        ACST     10.0       		% hz. Angle const set to 10 cc

*ZEND

P1	100

P3	200

*PLR3D

P1   100  100  10       TH 0.07      	% HA ZD DI, target height different for this observation

* TSTN	P4	LTD1    ROT3D     		% 3D orientation to calculate, IH fixed to 0.0

*V0

*PLR3D

P1   100   100   10

P2   101   101   11

P3   202   102   12

* TSTN	P1	Theo1           			% 3D orientation to calculate, IH fixed to 0.0

*V0

*ZEND

P2	100      OBSE  10   	% specific obs s.e. set

*DIST

P2	10				% uses default target

P3	20	TRGT T2		% new target and default parameters for the target

P4	30				%  uses the same target as the one above

*PLR3D					% uses default target again

P2   100   100   10   TRGT   T2   ZSE  8    	% new target and default parameters for the target

P3   200   200   20			%  uses the same target as the one above

* TSTN	P2	Theo1      IHFIX     IH  0.19    		% instrument height fixed and value set

*V0

*DIST

P1	10	OBSE 1.0   PPM  1.0   	% specific error data set

P3	20

P4	20

* TSTN	P3	Theo 1    IHFIX    TRGT  T2       		% target set to be T2

*V0

*DIST

P1	10

*ECHO 	  Ecarto1					% new stretched wire with each keyword

P3	0.25

P2          0.10

P1          0.05

*END